How to setup a S2S VPN connection the 10M way

Made by following this guide https://blogs.technet.microsoft.com/jletsch/2016/03/15/lets-configure-azure-site-to-site-vpn-with-rras-in-azure-resource-manager/

- Setup the server that will host the Routing and Remote access feature

- Install the Remote Access server role

- Click next on features, no additional features need to be added

- Click next on the Remote Access tab

- Check all 3 boxes in role services.

  - When prompted, use the default and make sure “Install management tools” is checked as well.

  - Click Next

- Install the IIS server role, accept the defaults it gives, then confirm.

  - Reboot the server once the roles are installed.

- Create an azure VNET with 10.1.1.0/24 (example IP, other sizes will work)

- Create a Gateway Subnet within the address space of the Vnet, atleast /28 in size

- Create a Virtual Network Gateway

- Create the local network gateway

  - The IP Address will be the IP for the on-prem gateway

  - The address space will be the address space for you on prem network or on prem virtual network

- In your local network gateway, go to connections, and add a new connection

  - Define your preshared key in the connection blade. Make note of this key, you will need it again soon.

Return to the on prem server

- Open the routing and remote access tool. Right click on the server name then click Configure Routing and Remote Access Server

- Choose “Custom Configuration”

- Choose “Demand Dial” then finish, it may ask you to start the service.

- If the Demand Dial interface does not popup on its own, right click network interfaces and choose “New demand dial interface” name it whatever you want.

  - Choose Connect using VPN

  - Choose Ikev2 as the vpn type

  - The destination address must be the Virtual Network Gateway Public IP address that you created earlier

  - Choose Route IP packets and interface

- If not prompted after completing the demand dial setup, click the drop down on IPV4 in the Routing and Remote Access tool. Right click static Routes, and add a new static route as shown.

  - The destination will be the address space of your Azure VNET

  - The network will be the network mask of your Azure VNEt

  - The metric will be the CIDR range of the Azure VNet (/24)

  - After clicking ok then next, you will be prompted to add credentials

  - Make the user name “Azure”, leave the rest blank and press next then finish

- Next, click on network interface under the server name. Choose your new Demand Dial interface that you created earlier, right click and choose properties.

- Enter the PreShared Key that you defined earlier in the Azure S2S connection.

- Then right click the demand dial interface again and connect, if you do not receive an error then it has worked.

- You can test your connection by creating a VM that is in your Azure VNET. Then remote to the VMs private IP address from your RRAS server.