10th Magnitude Network Discovery-Reply

- Describe existing network architecture

Current architecture (Item1)

LAN

Nexxus 7009-Dual Core  4(VDC), Floors, WAN,Datacenter and ADMIN

Nexxus 5548- 2(storage)  2(Hadoop)  2(UCS servers)

Floor distribution switches WS-C2960-40

UCS servers/Dell servers/HPE servers

WAN

DMVPN for INTL-20 sites

MPLS/AVPN For Azure

AVPN for Domestic offices-some sites

ATT NOD- Domestic offices-some sites

COX Tlan- Domestic offices-some sites

Verizon APN-Private.Camps and backup for domestic offices

ATT APN-Private. Camps and backup for domestic offices

Topology diagram included(Item2)

ROC/DR site (item 3)

Bandwidth- 1G NOD circuit ATT

1G COX Tlan

50MB Clink

DMZ located in Palo Firewall (item4)

IPS/IDS available onsite through security team(item5)

Cisco hardware.Palo Alto hardware, and HPE/NETAPP/UCS(item6)

IP addresses used (item7)

10.0.0.0/8 LAN/WAN

172.16.16.0/20 DMZ

192.168.0.0/16 Miscellaneous

Networking tools(item 8)

Gigamon for Data Captures/TAP

Network Observer/Netscout-Sniffers

Solarwinds- Network Management

Network Segmentation (item9)

Application and database servers use 10.10.7.0-10.10.11.0

No specific segments allocated, we try and have Database servers

On same segment as App Server if possible)

AccessRules-Firewalls(item 10)

There is no EAST/WEST segmentation through a firewalls

Access-Rules are ACL’s in Core/routers/LAN-WAN

DMZ is in the Firewall-

CDN (item11)

I assume you mean content delivery network

I,m sure we use AmazonAWS and other

We have built an Internal CDN for distribution to our rigs/Offices

DNS ZONES (item12)

We 3-4 Zones that cover the enterprise. WE are AD integrated with DNS.

There are separate DNS servers in Domestic offices and RIG Hub

3-4 Local DNS servers that service the Enterprise

We front end our DNS with Cisco Umbrella(OPENDNS)

Dynamic Routing protocols (item 13)

BGP for AVPN and Internet

OSPF Internal LAN

EIGRP Internals LANS

WAN accelerators (item 14)

None